INSTRUCTION MANUAL

2L/3.6L/4L/5L/8L/10L Mini Keg with straight Spear Tap co2

Regulator System

Before using the Mini Keg Tap System,become familiar

with the various parts.Wash the spear valve, mini keg,

faucet, silicone tubes in warm, soapy water.Rinse and dry

thoroughly.

①Mini keg Growler 2L/3.6L/4L/5L/8L/10L

②SS Straight Spear Valve

③Gas-In Connection Port

④Pressure Release Valve 30PSI

⑤cross-Bar

⑥co2 Regulator Knob

⑦30PSl Pressure Gauge

⑧Regulator Base

⑨Aluminium co2 Regulator Body

⑩Black Plastic Tap Handle

⑪chrome Plated Faucet Body

⑫silicone Beer Hose

⑬16g Threaded co2 cartridge/Capsule(Not Included)

Thread:3/8""-24 UNF

⑭ Copper swivel joint

PART ONE:HOW TO ASSEMBLY

- Fill mini keg at your favourite craft breweries or with

- your own home brew beer.

- cut beer hose according to keg height and attach onto

- barbed fitting of spear valve, make sure hose can touch

- bottom of keg without overlong or over short.

- Insert spear valve into keg and screw on until tight.

4.Check whether the regulator knob is in OFF position

(Decrease). If not, please follow arrow mark instruction

to turn the regulator knob counter-clockwise to off position.

5.Turn in a new 16g co2 capsule with thread type of 3/8-24UNF, turn clockwise firmly into the regulator base.

REMARK: DON'T stop turning if you hear a slight gas leak

swish, finish turning it in all the way it seal properly.

- Tighten the copper adapter, then Connect the regulator with capsule to the spear valve gas-in port .

- Install faucet onto spear valve cross-bar, make sure

- the connection is proper and tight.

REMARK: Improper  combination would cause leakage of beer.

- Pull on pressure release valve ring for a couple

- of seconds, if the keg was shaken in transit or warmed,

- it may contain excessive pressure which must be relieved before tapping.A hissing noise is expected.

- Slowly turn the regulator

- knob in clockwise direction to3-5 PSI, and stop for 2

- seconds ad turn off

- regulator. Wait for 2mins ,then pull on the

- pressure release valve ring

- fora couple of seconds.This

- is for adjusting pressure of keg beer.

- Put a glass or pitcher under the faucet spout, with the

- faucet knob pulled forward(ON), slowly turn the

- regulator knob in clockwise direction, until beer starts to

- flow out.

- REMARK: Don't continue adding pressure, suggest

- tapping 5-8 PSI(Max pressure 25PSI), so please stop

- turning the regulator knob when the pressure is enough

- to tap beer out with perfect foam.Too much pressure

- would cause over foam. When the beer stopped or flow

- slower, try turning the regulator knob on slowly, to get

- bigger pressure.

- when you finish tapping,please turn regulator knob

- to OFF position.

- If no beer flow out, then the co2 capsule needs to be

- replaced, unless no beer is remained(Normally, 16g

- capsule can tap 4-5 liters of beer).

- caution: Removing a punctured capsule from regulator should be done slowly.Swish is expected if pressure remains.

- Turn the regulator knob off and install a new capsule

- into the regulator by following above steps.

- PART TWO: TROUBLE SHOOTING GUIDE

- 1.Faucet Leakage:

- Solution: please screw out and remove the faucet, and

- install again, make sure the faucet can match the

- connection of spear valve cross-bar tightly.

- 2.Too much Foam:

- Solution: Turn off the regulator and pull the pressure

- release valve ring to release the pressure.And turn on

- the regulator again and adjust to 3-5psi.

- 3.Leakage of co2:

- Co2 capsule should be installed properly and tightly. In

- case there is co2 leakage,please observe the position of

- leakage and continue with the tightening process, but

- never screw out if gas remains.

- PART THREE: IMPORTANTSAFETYINFORMATION

- To reduce the risk of personal injury or property damage

- when using this appliance, basic safety precautions

- should always be observed, including the following:

- 1.Keep away from children under 18.

- 2.Never point cylinder towards someone's face.

- 3.Do not heat over140°F as rupture may occur!

- 4.Discontinue using of this equipment if unsolvable

- leakage or visible damage is found.

- 5.Capsule end become punctured when turned into

- regulator base.Unscrewing before it's empty can result in loss of high pressure gas.Please remember, never unscrew the co2 capsule before gas used up, liquid co2 spray onto unprotected skin may cause freezing burns.

- 6.Exterior of capsule may become frozen when in use,

- this is normal.But don't touch with bare hands, your

- fingers could stick to the frozen surface and cause

- frostbite.

- 7.Please ship via land or sea shipment, never bring co2

- capsule when you travel by air.